Standard Operating Procedure

Crusher and Screen

ODISHA MINING CORPORATION LTD.

KODINGAMALI BAUXITE MINE

Authorized By

- PURPOSE:

To provide the Standard Operating Procedure for Fixed Crusher operation in Kodingamali Bauxite Mine.

- SCOPE:

This procedure is applicable for safe Operation of Fixed Crusher in Kodingamali Bauxite Mine.

- RESPONSIBILITY:

|  | Fixed Crusher Operation | Crusher Incharge |
| --- | --- | --- |
|  | Fixed Crusher Maintenance | Crusher Incharge |
|  | Crushing – Conveying System Operation | Supervisor/Shift Incharge |
|  | Operational Control | Area in-charge |

Crusher in charge shall be responsible for implementation of this standard.

- SAFETY AND HEALTH HAZARDS:

- Slip/trip/fall

- Head on Collision

- Roll Over/Toppling of Dumper

- Hitting by other HEMM/Structure/Persons

- Dust generation during Operation

- Noise during Operation

- Vibration during Operation

- Diesel/ Hydraulic Oil - Inflammable, Fire hazards.

- Acid Batteries- Skin contact can lead to extensive and severe burns and highly toxic.

In order to avoid injuries to personal and damage to machinery, the following precautions are very important.

- Work required PPE i.e. with safety shoes, helmet, dust musk, fluorescent jacket suitable for long distance visibility etc.

- Adherence to instruction related to Operating safety as per statutory guidelines.

- Pre Operation Safety features checklist.

- Regular Checks & Maintenance.

Note: Report immediately to Supervisor, Shift Incharge and Safety officer for any incident/accident.

- ENVIRONMENT ASPECT:

- Dust generation

- Noise Pollution

- Vibration

- Oil Spillage

- Diesel- Inflammable, Fire hazards.

- Electrical – Fire hazards

- Oil/Grease – Inflammable, Fire hazards.

- Acid Batteries- Skin contact can lead to extensive and severe burns and highly toxic.

Preventive measures:

- Use of Dry fog system for dust control.

- Use of Dust Mask/Earplug

- Control of leakages

- TOOLS:

- All Work required PPEs like Safety Helmet, Safety shoes, Nose mask, Ear Plug & Jacket.

- Required Tools & Tackles

- PROCEDURE:

  - General Instructions for Fixed Crusher Operation:

- Ensure the use of all required personal protective equipment by all persons.

- Before starting the crusher, always give a warning signal.

- During operation always keep the crusher control room doors closed and no one in the vicinity of the room.

- Ensure no person working therein, when Crusher in operation and material is unloading.

- Persons working in crusher hopper should always wear all PPE’s as per requirements.

- Safety belts should be used during working above crusher openings and lifeline should be tied sufficiently short to prevent wearer to fall into crusher.

- Before doing repair work on crusher take work permit and all motors should be shut down and locked and rendered inoperative and tagged to prevent any other person from starting.

- Take adequate precautions while cleaning of material in crusher hopper, chutes.

- No person should clean or repair any part while belt conveyor in motion.

- Keep the path and railing along belt conveyor clean and in good condition.

- No person should travel or ride moving belt conveyor, use over bridge to cross the conveyor even if belt is not running.

- Take work permit before working on belt conveyor.

- Ensure all guards of machine, couplings are in place after any repair work.

- Always be cautious at work.

  - Normal Start up:

- Before starting the equipment in sequence, ensure no person is engaged in any cleaning, repair/rectification work.

- Remove the barricade from hopper.

- Give clear warning signal before starting the system and at the time of restarting after intermittent stoppage.

- When the system is stopped due to any reason and restarted after sometime ensure that no one is working on any part of the equipment.

- Whenever any repair/rectification of work has to be carried out in any equipment/system in running condition, ensure personal supervision.

- When there is no material being received from mines due to any reason, stop all auxiliaries – it saves precious “POWER”.

- Roles and Responsibility

  - Duties of Crusher Shift Incharge:

- Crusher shift in charge shall be responsible for maintenance and safe operation of crusher.

- He shall ensure pre shift maintenance of crusher and shall maintain the record.

- He shall inspect and ensure if all the staff are wearing PPE’s

- He shall ensure proper demarcation of site if any

  - Duties of operator:

Operator shall check the following:

- Crusher liner plate bolts for tightness.

- Material accumulation under discharge belt.

- Condition of chain scraper.

- Compressor oil level and V-belt condition.

- Apron feeder spill back hopper.

- Crusher and hopper condition for any abnormality.

- Spillage from crusher housing.

- Warning hooter and lights.

- Discharge chute.

- Emergency switches start or stop.

- Material pile up on pulleys and idlers or below idlers and belts.

- Lumps of materials are not wedged between belt and pulley.

- Conditions of all chutes, gear box not overheating and not producing any   unusual noise.

- Idler, self-aligning idlers rotating freely and not missing along the length of belt.

- Condition of couplings, idler supports, rollers and belt tensioning rope.

- Condition of belt scrapers, skirt boards are not damaging belt, condition of pulley lagging.

- Belt is running centrally and not slips condition of splices and joints.

- All the safety devices like pull cord etc. are not damaged and in working condition

- Water tank and chemical tank level of dust suppressions system.

- In case any fault in any of the system that would require more than 15 minutes to restore back in operation, then stop the main drive also along with other auxiliaries.

- Communicate continuously with the Mines Pit and BC-5 Discharge chute and loading point for smooth and continuous operation.