Standard Operating Procedure

Excavator Operation

ODISHA MINING CORPORATION LTD.

KODINGAMALI BAUXITE MINE

Authorized By

- Purpose: This procedure is to establish and ensure safe operation for Excavator operating in mines.

- Responsibility:

Mechanical in charge shall be overall in charge to ensure safety of HEMM operating in mines. He shall work along with Shift Incharge, Area Incharge and Safety officer to implement the standard across mine.

Operator shall be responsible to ensure safe operation of vehicle as per standard.

| - PPE: - Safety helmet - Safety shoe - Rubber Gloves - Nose Mask - Fluorescent jacket - Safety goggles. | Training needs: - Line of Fire - Traffic rules and Driving safety - Usage of PPEs - Fire Safety in HEMM - Safety and Health Hazard - Excavator Operation - Loading and Hauling Operation |
| --- | --- |

- Safety and Health Hazard

- Slip/trip/fall.

- Head on Collision.

- Roll Over/Toppling of water tanker.

- Hitting to other Truck/Structure/Persons.

- Dust generation during Operation.

- Noise during Operation.

- Vibration during Operation.

- Diesel/ Hydraulic Oil - Inflammable, Fire hazards.

- Acid Batteries- Skin contact can lead to extensive and severe burns and highly toxic.

In order to avoid injuries to personal and damage to machinery, the following precautions are very important.

- Work required PPE i.e. with safety shoes, helmet, dust musk, fluorescent jacket suitable for long distance visibility etc.

- Pre Operation Safety features checklist.

- Regular Checks & Maintenance.

- Use seat belt while driving.

- Procedure:

- Operator is equipped  with  full  safety gadgets  needed  for his  operation (work)  and Operator  never starts  any  activities without equipping safety gadgets.

- Operator has conducted  full check up  on safety devices  and other parameters / vital components   for his allocated  excavator as per  operator manual /manufacturer  safety precaution s before starting the engine  and to execute any operation , Operator determines &  ensures  that  it is functioning in order / it is  in safe  working / serviceable condition.

- Vital   components  like:-

- The  hydraulic  system

- The boom

- The  cabin

- Ram and the Bucket

- Hydraulic  Jack

- Sharp teeth of the  bucket

- Two control levers (lock / unlock)- control virtual all movements of the excavator.

- Upper body of the excavator which swivels trough 360 degree by rotating the slewing ring.

- Amongst  the important  components of the  excavators hydraulic system are its :-

- Filters

- Pipes

- Hoses

- Seals

- Valves

- Cylinders & oil

- Safety  devices  and  parameters  like:-

- Horn  and  light

- All   gears  & control  levers

- Lock & Unlock controls

- Hydraulic pump

- Bucket /Boom/ cylinder movement.

- Level (quantity) of HSD in fuel tank.

- Level (quantity) of hydraulics.

- Level(quantity /quality) of coolant  and  brake oil

- All pressures & temperatures parameters (while checking or operating).

- Operator will ensure that there is no man within the range of movement of hydraulic components of the machine / immediate range of the attachment of equipment. Operator will not allow to any rank of man/men in close proximity to the bucket either it is cutting /digging or any operation.  Several  times  accidents  have taken place on account of above  points Because it has been found that :-- WHEN BUCKET SLIPS  IT MAY SWING  TO  SIDES (LEFT OR RIGHT)  this causes by  combination of  accumulated  tension  in the  arm of the hydraulic excavator and  slight delay  before the operator  could react  and stop the  bucket. In this context it does not appear nothing is mechanically wrong nor did the operator to make an error or misunderstand a hand signal.

- Heavy equipment’s to be operated minimum 1.5 meter from the edge of trenches / excavations.

- Operator will ensure that:--

- While Operator gives load (minerals / material) to hauling equipments / Crusher or Screen, he always tries to attain full fill factor of bucket.

- After giving  recommended  load ( with respect to the capacity of hauling equipment), he     gives  appropriate  signals( horns) to hauling equipment  operator for proceeding  his  vehicle .

- Operator will give appropriate signals (horn) to dumper operators / truck operators to place their      vehicle in proper range & direction to give loads safely.

- While Operator gives  loads ( mineral /material )  to  Crusher Hopper / Screen  Hopper, Operator will ensure that  he  operate bucket in such way which  does allow to promote spillage below the hopper .

- Operator will ensure that he does not make delay deliberately while he operate excavator machine.

- Operator will ensure that he never ignore any abnormalities are sensed while machine is in operation and he conveys the message to his supervisor / concerned authorities.

- Operator will ensure that  he follows all traffic  rules  applicable to mines / mines premises , Mines safety and   warning  signal  while, he shifts  /  marches  machine to another work place

- Operator will ensure that he uses horns to alert the traffic coming from other side during shifting / marching   excavator to another work place, whenever sight is intersected.

- Operator will ensure that while negotiating  a  curve, he maintains  low speed  and  select a turning radius commensurate with speed  and  to adjust  crawling  component of machine.

- While operates up or down gradient, he keeps machine in low gear range (by controlling levers) to avoid engine over race and consequent damage.

- Operator will ensure that there is no leakage from hoses / connections while he operates excavator.

- Operator will ensure that filters of the hydraulic system is working smoothly, no cavitation is producing during operation.

- Filters are filtrate regularly / replaced as applicable with running hours   of machine.

- Operator will try to ensure the availability through his supervisor / maintenance personnel  (spares) at least  one  piece of  pipes, hoses & seals  which  becomes  important sometimes  as hydraulic hoses may brust   under pressure. And connection of hoses to machine parts is the most vulnerable points.

- Operator ensures to perform   his duty as detailed in Job description for his category.

- Operator ensures to obey instructions passed to him by his supervisor /superior/authorities.

Note: Report immediately to Supervisor, Shift Incharge and Safety officer for any incident/accident.