Standard Operating Procedure

Welding and Cutting Preparation

ODISHA MINING CORPORATION LTD.

KODINGAMALI BAUXITE MINE

Authorized By

- Scope and Field of Application: Procedures establish under this standard is for safe operation during welding and cutting preparation.

- Accountability: Mechanical I/C

| - PPE: - Safety helmet - Safety shoe - Nose Mask - Safety goggles. - Welding Glass - Face shield - Flame resistant gloved | Tools: - Vas tools - Pliers - Channel lock - Electric Angle Grinder - Circular Saw - Reciprocating Saw - Torch Kit - Tape - Welding Cylinders | Training needs: - Line of Fire - Work at height - Electrical Safety - Usage of PPEs - Safe welding operation |
| --- | --- | --- |

- Responsibility: it shall be the responsibility of mechanical in charge, who shall delegate a workshop in charge. Both shall be responsible for ensuring implementation of this standard.

- Procedure:

- Carry out welding and flame cutting on the plant only by the authorized persons.

- Ensure that all required PPE is in good condition and correctly fitted.

- Before carrying out welding and flame cutting operations, clean the plant and its surroundings from dust and other inflammable substances and make sure the premises are adequately ventilated.

- Ensure there should not be any damage to welding cable, welding holder, etc.

- Ensure use of a separate welding cable for earthling up to the welding location.

- Check for leaks and faults if it is require change the hose pipe and cylinders, never use in leakage condition.

- Never test for leaks with a flame.

- Connect the flashback arrester to protect cylinders from flashbacks and back fires.

- Ensure that, cylinder valve and regulators are clean & dry. No oil and grease are stacked to the valve and regulators.

- Avoid standing on the hoses or letting it twisted as this may shut off the gas and cause trouble.

- Never light the torch while both the valves are wide open.

- Clean the place which is to be welded.

- Allow the welded parts to cool for same time.

- Always keep the adjusting valve on the regulator closed when not in use.

- Keep all cylinders away from heat sources.

- Keep flammable (Acetylene & LPG) gases away from any source of ignition (NB: any electrical switch is an ignition source).

- Storage areas should secure against unauthorized entry.

- Full and Empty cylinders should be stored separately if space permits.

- Cylinders should not be stored for a long time, rotate stock.